Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| S1 | VLAN 1 | 10.10.10.2 | 255.255.255.0 |
| PC1 | NIC | 10.10.10.10 | 255.255.255.0 |
| PC2 | NIC | 10.10.10.11 | 255.255.255.0 |
| Rogue Laptop | NIC | 10.10.10.12 | 255.255.255.0 |

Objective

Part 1: Configure Port Security

Part 2: Verify Port Security

Background

In this activity, you will configure and verify port security on a switch. Port security allows you to restrict a port’s ingress traffic by limiting the MAC addresses that are allowed to send traffic into the port.

Read me first:

Configure Port Security

Access the command line for S1 and enable port security on Fast Ethernet ports 0/1 and 0/2.

S1(config)# interface range fa0/1 - 2

S1(config-if-range)# switchport port-security

Set the maximum so that only one device can access the Fast Ethernet ports 0/1 and 0/2.

S1(config-if-range)# switchport port-security maximum 1

Secure the ports so that the MAC address of a device is dynamically learned and added to the running configuration.

S1(config-if-range)# switchport port-security mac-address sticky

Set the violation so that the Fast Ethernet ports 0/1 and 0/2 are not disabled when a violation occurs, but a notification of the security violation is generated and packets from the unknown source are dropped.

S1(config-if-range)# switchport port-security violation restrict

Disable all the remaining unused ports. Hint: Use the range keyword to apply this configuration to all the ports simultaneously.

S1(config-if-range)# interface range fa0/3 - 24 , gi1/1 – 2

Comment: instead of gi1/1 – 2, it may be gi0/1 – 2

S1(config-if-range)# shutdown

Verify Port Security

From PC1, ping PC2.

Verify port security is enabled and the MAC addresses of PC1 and PC2 were added to the running configuration.

Attach Rogue Laptop to any unused switch port and notice that the link lights are red.

Enable the port and verify that Rogue Laptop can ping PC1 and PC2. After verification, shut down the port connected to Rogue Laptop.

Disconnect PC2 and connect Rogue Laptop to PC2’s port. Verify that Rogue Laptop is unable to ping PC1.

Display the port security violations for the port Rogue Laptop is connected to.

- S1# show port-security interface fa0/2

Disconnect Rouge Laptop and reconnect PC2. Verify PC2 can ping PC1.

Why is PC2 able to ping PC1, but the Rouge Laptop is not? The port security that was enabled on the port only allowed the device, whose MAC was learned first, access to the port while preventing all other devices access.