Lab 2 - Switch Settings

Topology

Addressing Table

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC1 | 192.168.1.1 | 255.255.255.0 |
| PC2 | 192.168.1.2 | 255.255.255.0 |

Objectives

Verify the Default Switch Configuration

Configure a Basic Switch Configuration

Configure a MOTD Banner

Configure the PCs

Save Configuration Files to NVRAM

Background

In this lab, you will first perform basic switch configurations. Then, you will implement basic connectivity by configuring IP addressing on PCs. You will use various show commands to verify configurations and use the ping command to verify basic connectivity between PCs. You will secure access to the command-line interface (CLI) and console ports using encrypted and plain text passwords.

Verify the Default Switch Configuration

Enter privileged EXEC mode.

You can access all switch commands from privileged EXEC mode. However, because many of the privileged commands configure operating parameters, privileged access should be password-protected to prevent unauthorized use.

The privileged EXEC command set includes those commands contained in user EXEC mode, as well as the configure command through which access to the remaining command modes are gained.

Click S1 and then the CLI tab. Press Enter.

Enter privileged EXEC mode by entering the enable command:

Switch> enable

Switch#

Notice that the prompt changed in the configuration to reflect privileged EXEC mode.

Examine the current switch configuration.

Enter the show running-config command.

Switch# show running-config

Answer the following questions:

How many FastEthernet interfaces does the switch have?

How many Gigabit Ethernet interfaces does the switch have?

Which command will display the current contents of non-volatile random-access memory (NVRAM)?

Why does the switch respond with startup-config is not present?

Create a Basic Switch Configuration

Assign a name to a switch.

To configure parameters on a switch, you may be required to move between various configuration modes. Notice how the prompt changes as you navigate through the switch.

Switch# configure terminal

Switch(config)# hostname S1

S1(config)# exit

S1#

Secure access to the console line.

To secure access to the console line, access config-line mode and set the console password to letmein.

S1# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

S1(config)# line console 0

S1(config-line)# password letmein

S1(config-line)# login

S1(config-line)# exit

S1(config)# exit

%SYS-5-CONFIG_I: Configured from console by console

S1#

Why is the login command required? Verify that console access is secured.

Exit privileged mode to verify that the console port password is in effect.

S1# exit

Switch con0 is now available

Press RETURN to get started.

User Access Verification

Password:

S1>

Note: If the switch did not prompt you for a password, then you did not configure the login parameter in Step 2.

Secure privileged mode access.

Set the enable password to c1$c0. This password protects access to privileged mode.

Note: The 0 in c1$c0 is a zero, not a capital O. This password will not grade as correct until after you encrypt it in Step 8.

S1> enable

S1# configure terminal

S1(config)# enable password c1$c0

S1(config)# exit

%SYS-5-CONFIG_I: Configured from console by console

S1#

Verify that privileged mode access is secure.

Enter the exit command again to log out of the switch.

Press <Enter> and you will now be asked for a password:

User Access Verification

Password:

The first password is the console password you configured for line con 0. Enter this password to return to user EXEC mode.

Enter the command to access privileged mode.

Enter the second password you configured to protect privileged EXEC mode.

Verify your configurations by examining the contents of the running-configuration file:

S1# show running-config

Notice how the passwords are in plain text. This could pose a security risk if someone is looking over your shoulder.

Configure an encrypted password to secure access to privileged mode.

The enable password should be replaced with the newer encrypted secret password using the enable secret command. Set the enable secret password to itsasecret.

S1# config t

S1(config)# enable secret itsasecret

S1(config)# exit

S1#

Note: The enable secret password overrides the enable password. If both are configured on the switch, you must enter the enable secret password to enter privileged EXEC mode.

Verify that the enable secret password is added to the configuration file.

Enter the show running-config command again to verify the new enable secret password is configured.

Note: You can abbreviate show running-config as

S1# show run

What is displayed for the enable secret password?

Why is the enable secret password displayed differently from what we configured?

Encrypt the enable and console passwords.

As you noticed in Step 7, the enable secret password was encrypted, but the enable and console passwords were still in plain text. We will now encrypt these plain text passwords using the service password-encryption command.

S1# config t

S1(config)# service password-encryption

S1(config)# exit

If you configure any more passwords on the switch, will they be displayed in the configuration file as plain text or in encrypted form? Explain..

Configure a MOTD Banner

Configure a message of the day (MOTD) banner.

The Cisco IOS command set includes a feature that allows you to configure messages that anyone logging onto the switch sees. These messages are called message of the day, or MOTD banners. Enclose the banner text in quotations or use a delimiter different from any character appearing in the MOTD string.

S1# config t

S1(config)# banner motd "This is a secure system. Authorized Access Only!"

S1(config)# exit

%SYS-5-CONFIG_I: Configured from console by console

S1#

When will this banner be displayed?

Why should every switch have a MOTD banner?

Save Configuration Files to NVRAM

Verify that the configuration is accurate using the show run command.

Save the configuration file.

You have completed the basic configuration of the switch. Now back up the running configuration file to NVRAM to ensure that the changes made are not lost if the system is rebooted or loses power.

S1# copy running-config startup-config

Destination filename [startup-config]?[Enter]

Building configuration...

[OK]

What is the shortest, abbreviated version of the copy running-config startup-config command?

Examine the startup configuration file.

Which command will display the contents of NVRAM?

Are all the changes that were entered recorded in the file?

Configure S2

You have completed the configuration on S1. You will now configure S2. If you cannot remember the commands, refer to Parts 1 to 4 for assistance.

Configure S2 with the following parameters:

- Name device: S2

- Protect access to the console using the letmein password.

- Configure an enable password of c1$c0 and an enable secret password of itsasecret.

- Configure a message to those logging into the switch with the following message:

Authorized access only. Unauthorized access is prohibited and violators will be prosecuted to the full extent of the law.

- Encrypt all plain text passwords.

- Ensure that the configuration is correct.

- Save the configuration file to avoid loss if the switch is powered down.

Configure the PCs

Configure PC1 and PC2 with IP addresses.

  - Configure both PCs with IP addresses.

Click PC1 and then click the Desktop tab.

Click IP Configuration. In the Addressing Table above, you can see that the IP address for PC1 is 192.168.1.1 and the subnet mask is 255.255.255.0. Enter this information for PC1 in the IP Configuration window.

Repeat steps 1a and 1b for PC2.

## Test connectivity to switches.

Click PC1. Close the IP Configuration window if it is still open. In the Desktop tab, click Command Prompt.

Type the ping command and the IP address for PC2 and press Enter.

Packet Tracer PC Command Line 1.0

PC> ping 192.168.1.2

The lab work is part of ‘Configuring Initial Switch Settings‘ activity from Cisco CCNA course. All rights reserved.