The ARP Table

Topology

Addressing Table

| Device | Interface | MAC Address | Switch Interface |
| --- | --- | --- | --- |
| Router0 | Gg0/0 | 0060.7062.CEB5 | G0/1 |
|  | S0/0/0 | N/A | N/A |
| Router1 | G0/0 | 0040.9AE4.B970 | G0/1 |
|  | S0/0/0 | N/A | N/A |
| 10.10.10.2 | Wireless | 00E0.8F64.CC37 | F0/2 |
| 10.10.10.3 | Wireless | 0060.47C3.9D64 | F0/2 |
| 172.16.31.2 | F0 | 0001.42E3.7BEA | F0/1 |
| 172.16.31.3 | F0 | 0060.702C.3A11 | F0/2 |
| 172.16.31.4 | G0 | 00E0.F7E7.ADE3 | F0/3 |

Background

This activity is optimized for viewing PDUs. The devices are already configured. You will gather PDU information in simulation mode and answer a series of questions about the data you collect.

Ping is a computer network administration software utility used to test the reachability of a host on an Internet Protocol (IP) network.

Ping operates by sending Internet Control Message Protocol (ICMP) echo request packets to the target host and waiting for an ICMP echo reply.

Examine an ARP Request

Generate ARP requests by pinging 172.16.31.3 from 172.16.31.2 (Steps 1 and 2 of Part 1 include an example of local communication; that is, communication within the same network of 172.16.31.0/24)

    - Click 172.16.31.2 and open the Command Prompt.

    - Enter the arp -d command to clear the ARP table.

    - Enter Simulation mode and enter the command ping 172.16.31.3. Two PDUs will be generated. The ping command cannot complete the ICMP packet without knowing the MAC address of the destination. So the computer sends an ARP broadcast frame to find the MAC address of the destination.

    - Click Capture/Forward once. The ARP PDU moves Switch1 while the ICMP PDU disappears, waiting for the ARP reply. Open the PDU and record the destination MAC address. Is this address listed in the table above? No; an all ‘1’ MAC address is used instead (all ‘1’ is FFFF.FFFF.FFFF) which is the broadcast MAC address.

    - Click Capture/Forward to move the PDU to the next device. How many copies of the PDU did Switch1 make? 3 –to all ports of the switch except the one from which the packet was received (remember: we are dealing with a broadcast!)

    - What is the IP address of the device that accepted the PDU? 172.16.31.3; it is the IP of the device of which we require its MAC address

    - Open the PDU and examine Layer 2. What happened to the source and destination MAC addresses? Source became destination, FFFF.FFFF.FFFF turned into MAC address of 172.16.31.3

    - Click Capture/Forward until the PDU returns to 172.16.31.2. How many copies of the PDU did the switch make during the ARP reply? 1; only the device with the sought MAC address replies

Examine the ARP table.

    - Note that the ICMP packet reappears. Open the PDU and examine the MAC addresses. Do the MAC addresses of the source and destination align with their IP addresses? Yes (as in the addressing table; otherwise or if in doubt, you should discuss the matter with your instructor)

    - Switch back to Realtime and the ping completes.

    - Click 172.16.31.2 and enter the arp –a command. To what IP address does the MAC address entry correspond? 172.16.31.3

In general, when does an end device issue an ARP request? When it does not know the receiver’s MAC address.

Examine a Switch MAC Address Table

Generate additional traffic to populate the switch MAC address table. (In Steps 1 and 2 of Part 2, we examine MAC address tables for both switches for communication within the 172.16.31.0/24 network and within the 10.10.10.0/24 network)

    - From 172.16.31.2, enter the ping 172.16.31.4 command.

    - Click 10.10.10.2 and open the Command Prompt.

    - Enter the ping 10.10.10.3 command. How many replies were sent and received? For a 100% success rate; 4 sent, 4 received

Examine the MAC address table on the switches.

    - Click Switch1and then the CLI tab. Enter the show mac-address-table command. Do the entries correspond to those in the table above? Yes; if not or if you are in doubt: report the matter back to your instructor(s).Remeber: Switch 1 should know the devices (by their MAC address) connected to its ports!

    - Click Switch0, then the CLI tab. Enter the show mac-address-table command. Do the entries correspond to those in the table above? As in the question above, the answer should be Yes; but if not or if you are in doubt: report the matter back to your instructor(s)

    - Why are two MAC addresses associated with one port? Because both devices connect to one port through a wireless Access Point.

Generate traffic to produce ARP traffic (Steps 3 and 4 cover an example of using ARP to be able to communicate with a node in a remote network: form 172.16.31.0/24 network to the 10.10.10.0/24 network and vice versa)

    - Click 172.16.31.2 and open the Command Prompt.

    - Enter the ping 10.10.10.1 command.

    - Type arp –a. What is the IP address of the new ARP table entry? 172.16.31.1

    - Enter arp -d to clear the ARP table and switch to Simulation mode.

    - Repeat the ping to 10.10.10.1. How many PDUs appear? 2 we can distinguish an ICMP packet and an ARP packet.

    - Click Capture/Forward. Click the PDU that is now at Switch1. What is the target destination IP destination address of the ARP request? 172.16.31.1 – this is the IP of the default gateway (router)

    - The destination IP address is not 10.10.10.1. Why? The gateway address of the router interface is stored in the IPv4 configuration of the hosts. If the receiving host is not on the same network (on a remote network), the source uses the ARP process to determine a MAC address for the router interface serving as the gateway.

Examine the ARP table on Router1

    - Switch to Realtime mode. Click Router1 and then the CLI tab.

    - Enter privileged EXEC mode and then the show mac-address-table command. How many MAC addresses are in the table? Why? Zero, This command means something completely different than the switch command show mac address-table: a switching module can be used by a router, hence the command is allowed in router for that reason!

    - Enter the show arp command. Is there an entry for 172.16.31.2? Yes

    - What happens to the first ping in a situation where the router responds to the ARP request? It times out