Lab 6: Connecting Routers and LANs

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.10.1 | 255.255.255.0 | N/A |
|  | G0/1 | 192.168.11.1 | 255.255.255.0 | N/A |
|  | S0/0/0 (DCE) | 209.165.200.225 | 255.255.255.252 | N/A |
| R2 | G0/0 | 10.1.1.1 | 255.255.255.0 | N/A |
|  | G0/1 | 10.1.2.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 209.165.200.226 | 255.255.255.252 | N/A |
| PC1 | NIC | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC2 | NIC | 192.168.11.10 | 255.255.255.0 | 192.168.11.1 |
| PC3 | NIC | 10.1.1.10 | 255.255.255.0 | 10.1.1.1 |
| PC4 | NIC | 10.1.2.10 | 255.255.255.0 | 10.1.2.1 |

Introduction

In this lab, you will use various show commands to display the current state of the router. You will then use the Addressing Table to configure router Ethernet interfaces. Finally, you will use commands to verify and test your configurations.

Note: The routers in this activity are partially configured. Some of the configurations may not have been covered (thus far) in our lecture (for instance the routing table part), but are provided to assist you in using verification commands. The serial (connections) interfaces between routers R1 and R2 defines a WAN. The serial interfaces are already configured and active. EIGRP is a routing protocol; routing is already configured to use EIGRP in R1 and R2.

Display Router Information

Display interface information on R1.

Note: Click a device and then click the CLI tab to access the command line directly. The console password is cisco. The privileged EXEC password is class.

What does the command ‘show interfaces’ display about all configured interfaces of a router? Which command displays the statistics for all interfaces configured on a router? ‘show interfaces’ displays statistics about configured interfaces (and by the way, ‘show interface’ can be an abbreviation of the command!). The command can take further arguments for more precise statistics.

Use the command show interface serial 0/0/0 to display the information about the Serial 0/0/0 interface

By entering the command to display the statistics for the Serial 0/0/0 interface on R1,answer the following questions:

What is the IP address configured on R1? 209.165.200.225/30

What is the bandwidth on the Serial 0/0/0 interface in kbits? 1544 kbits

Now, enter the command to display the statistics for the GigabitEthernet 0/0 interface and answer the following questions:

What is the IP address on R1? There is no IP address configured on the GigabitEthernet 0/0 interface.

What is the MAC address of the GigabitEthernet 0/0 interface? 0060.3E1C.D633

What is the bandwidth on the GigabitEthernet 0/0 interface? 1000000 kbits

Display a summary list of the interfaces on R1.

Now use the command ‘show ip interface brief’ which displays only a brief summary of the current interfaces, statuses, and IP addresses assigned to them.

Enter such command on each router and answer the following questions:

How many serial interfaces are there on R1 and R2?. Each router has 2 serial interfaces

How many Ethernet interfaces are there on R1 and R2? R1 has 6 Ethernet interfaces and R2 has 2 Ethernet interfaces.

- 3) Are all the Ethernet interfaces on R1 the same? If no, explain the difference(s). No they are not! There are two Gigabit Ethernet interfaces and 4 Fast Ethernet interfaces. Gigabit Ethernet interfaces support speeds of up to 1,000,000,000 bits and Fast Ethernet interfaces support speeds of up to 1,000,000 bits.

Display the routing table on R1.

Use command ‘show ip route’ to display the content of the routing table (routine information)

Enter the command on R1 and answer the following questions:

How many connected routes are there (look for whenever the C code is used)? 1

Which route is listed? 209.165.200.224/30

Configure Router Interfaces

Configure the GigabitEthernet 0/0 interface on R1.

Enter the following commands to address and activate the GigabitEthernet 0/0 interface on R1:

R1(config)# interface gigabitethernet 0/0

R1(config-if)# ip address 192.168.10.1 255.255.255.0

R1(config-if)# no shutdown

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

It is good practice to configure a description for each interface to help document the network information. Configure an interface description indicating to which device it is connected.

R1(config-if)# description LAN connection to S1

R1 should now be able to ping PC1.

R1(config-if)# end

%SYS-5-CONFIG_I: Configured from console by console

R1# ping 192.168.10.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.10.10, timeout is 2 seconds:

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 0/2/8 ms

Configure the remaining Gigabit Ethernet Interfaces on R1 and R2.

Use the information in the Addressing Table

to finish the interface configurations for R1 and R2. For each interface, do the following:

Enter the IP address and activate the interface.

Configure an appropriate description.

Verify interface configurations.

Verify the Configuration

Use verification commands to check your interface configurations.

Use the show ip interface brief command on both R1 and R2 to quickly verify that the interfaces are configured with the correct IP address and active.

How many interfaces on R1 and R2 are configured with IP addresses and in the “up”? 2 on R1 and still 1 on R2; use the command ‘show ip interfaces brief’ or ‘show ip int brief’ on each router

What part of the interface configuration is NOT displayed in the command output – is the subnet shown for instance? The subnet mask is not shown

Test end-to-end connectivity across the network.

You should now be able to ping from any PC to any other PC on the network. In addition, you should be able to ping the active interfaces on the routers. For example, the following should tests should be successful:

From the command line on PC1, ping PC4. Router 2 interface linked to network 10.1.2/0/24 has not been configured; the ping command in this case is going to fail!

From the command line on R2, ping PC1. the ping command should be successful: the interface on R1 linked to network 192.168.10.0/24 is configured!

- Further work: Carry on configuring the routers’ interfaces according to the addressing table above. Issue ping commands to verify connectivity. Then issue the command ‘show ip route’ on each router to find out how many networks are directly connected to your routers; and how many routes are in the routing table. If in doubt, ask your lab demonstrator!