A Sub-netting Scenario

.

Topology

- Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.100.1 | 255.255.255.224 | N/A |
|  | G0/1 | 192.168.100.33 | 255.255.255.224 | N/A |
|  | S0/0/0 | 192.168.100.129 | 255.255.255.224 | N/A |
| R2 | G0/0 | 192.168.100.65 | 255.255.255.224 | N/A |
|  | G0/1 | 192.168.100.97 | 255.255.255.224 | N/A |
|  | S0/0/0 | 192.168.100.158 | 255.255.255.224 | N/A |
| S1 | VLAN 1 | 192.168.100.2 | 255.255.255.224 | 192.168.100.1 |
| S2 | VLAN 1 | 192.168.100.34 | 255.255.255.224 | 192.168.100.33 |
| S3 | VLAN 1 | 192.168.100.66 | 255.255.255.224 | 192.168.100.65 |
| S4 | VLAN 1 | 192.168.100.98 | 255.255.255.224 | 192.168.100.97 |
| PC1 | NIC | 192.168.100.30 | 255.255.255.224 | 192.168.100.1 |
| PC2 | NIC | 192.168.100.62 | 255.255.255.224 | 192.168.100.33 |
| PC3 | NIC | 192.168.100.94 | 255.255.255.224 | 192.168.100.65 |
| PC4 | NIC | 192.168.100.126 | 255.255.255.224 | 192.168.100.97 |

Scenario

In this activity, you are given the network address of 192.168.100.0/24 to subnet and provide the IP addressing for the network shown in the topology. Each LAN in the network requires enough space for, at least, 25 addresses for end devices, the switch and the router. The connection between R1 to R2 will require an IP address for each end of the link.

Design an IP Addressing Scheme

Subnet the 192.168.100.0/24 network into the appropriate number of subnets.

Based on the topology, how many subnets are needed? 5

How many bits must be borrowed to support the number of subnets in the topology table? 3

How many subnets does this create? 8

How many usable hosts does this create per subnet? 30

Note: If your answer is less than the 25 hosts required, then you borrowed too many bits.

Calculate the binary value for the first five subnets. The first subnet is already shown.

Net 0: 192 . 168 . 100 .  0   0   0   0   0   0   0   0

Net 1: 192 . 168 . 100 . ___ ___ ___ ___ ___ ___ ___ ___

Net 1: 192 . 168 . 100 .  0   0   1   0   0   0   0   0

Net 2: 192 . 168 . 100 . ___ ___ ___ ___ ___ ___ ___ ___

Net 2: 192 . 168 . 100 .  0   1   0   0   0   0   0   0

Net 3: 192 . 168 . 100 . ___ ___ ___ ___ ___ ___ ___ ___

Net 3: 192 . 168 . 100 .  0   1   1   0   0   0   0   0

Net 4: 192 . 168 . 100 . ___ ___ ___ ___ ___ ___ ___ ___

Net 4: 192 . 168 . 100 .  1   0   0   0   0   0   0   0

Calculate the binary and decimal value of the new subnet mask.

11111111.11111111.11111111. ___ ___ ___ ___ ___ ___ ___ ___

11111111.11111111.111111111. 1   1   1   0   0   0   0   0

255  .   255  .   255  . ______

255  .   255  .   255  .  224

Fill in the Subnet Table, listing the decimal value of all available subnets, the first and last usable host address, and the broadcast address. Repeat until all addresses are listed.

Note: You may not need to use all rows.

Subnet Table

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 192.168.100.0 | 192.168.100.1 | 192.168.100.30 | 192.168.100.31 |
| 1 | 192.168.100.32 | 192.168.100.33 | 192.168.100.62 | 192.168.100.63 |
| 2 | 192.168.100.64 | 192.168.100.65 | 192.168.100.94 | 192.168.100.95 |
| 3 | 192.168.100.96 | 192.168.100.97 | 192.168.100.126 | 192.168.100.127 |
| 4 | 192.168.100.128 | 192.168.100.129 | 192.168.100.158 | 192.168.100.159 |
| 5 | 192.168.100.160 | 192.168.100.161 | 192.168.100.190 | 192.168.100.191 |
| 6 | 192.168.100.192 | 192.168.100.193 | 192.168.100.222 | 192.168.100.223 |
| 7 | 192.168.100.224 | 192.168.100.225 | 192.168.100.254 | 192.168.100.255 |

Assign the subnets to the network shown in the topology.

Assign Subnet 0 to the LAN connected to the GigabitEthernet 0/0 interface of R1: 192.168.100.0 /27

Assign Subnet 1 to the LAN connected to the GigabitEthernet 0/1 interface of R1: 192.168.100.32 /27

Assign Subnet 2 to the LAN connected to the GigabitEthernet 0/0 interface of R2: 192.168.100.64 /27

Assign Subnet 3 to the LAN connected to the GigabitEthernet 0/1 interface of R2: 192.168.100.96 /27

Assign Subnet 4 to the WAN link between R1 to R2: 192.168.100.128 /27

Document the addressing scheme.

Fill in the Subnet Table using the following guidelines:

Assign the first usable IP addresses to R1 for the two LAN links and the WAN link.

Assign the first usable IP addresses to R2 for the LANs links. Assign the last usable IP address for the WAN link.

Assign the second usable IP addresses to the switches.

Assign the last usable IP addresses to the hosts.

Assign IP Addresses to Network Devices and Verify Connectivity

Most of the IP addressing is already configured on this network. Implement the following steps to complete the addressing configuration.

Configure IP addressing on R1 LAN interfaces

Configure IP addressing on S3, including the default gateway. (interface vlan 1)

Configure IP addressing on PC4, including the default gateway.

Further work: The subnet assigned to the WAN (link between the two routers) may be larger than necessary. The serial link only includes two devices. The 32-node subnet may be wasting IPs. You should change it to a network of 4 IP addresses only from which two IPs are usable!